# MAPATHON MASTER CHEAT SHEET

## GIS + Remote Sensing + AI/ML + QGIS + Google Earth Engine + Python

### Purpose

This document is a quick-reference guide for solving an unexpected GIS + AI/ML Mapathon problem.

Use it to quickly decide:

- What is the problem?

- What data is needed?

- Where can the data be obtained?

- Which tool should be used?

- Which GIS/remote-sensing method is appropriate?

- Which features or indices should be created?

- Is AI/ML actually required?

- How should the result be validated?

- What maps and outputs should be produced?

# 1. MAPATHON QUICK START

## Step 1 — Read the Problem Carefully

Extract:

- Problem

- Objective

- Study area

- Target feature

- Time period

- Required output

- Expected accuracy

- Available data

- Constraints

- Target users

- Final decision/application

### Ask:

What exactly do we need to identify, measure, predict, compare, or map?

## Step 2 — Identify the Problem Type

| Problem wording | Likely task |
| --- | --- |
| Detect | Detection / classification |
| Identify | Classification / spatial analysis |
| Map | Thematic mapping |
| Classify | Classification |
| Predict | Regression / classification |
| Estimate | Regression |
| Forecast | Time-series / prediction |
| Monitor | Time-series analysis |
| Assess | Spatial analysis / scoring |
| Risk | Risk mapping |
| Vulnerability | Vulnerability index / ML |
| Suitability | Multi-Criteria Decision Analysis |
| Prioritize | Ranking / weighted overlay |
| Change | Change detection |
| Expansion | Change detection |
| Growth | Change / temporal analysis |
| Hotspot | Hotspot / density analysis |
| Cluster | Clustering |
| Similarity | Similarity analysis |
| Segmentation | Image segmentation |
| Identify suitable locations | Suitability analysis |
| Find affected areas | Change/damage detection |
| Flood mapping | Sentinel-1 / water detection |
| Water detection | NDWI / MNDWI |
| Vegetation mapping | NDVI |
| Built-up mapping | NDBI |
| Burned area | NBR |
| Moisture | NDMI |
| Land-cover mapping | Classification |

# 2. CHOOSE THE SIMPLEST VALID METHOD

Use this decision rule:

Can GIS alone solve the problem?

|

YES

↓

Use GIS / Remote Sensing

|

NO

↓

Is statistical analysis sufficient?

|

YES

↓

Use statistical method

|

NO

↓

Use AI / ML

### Important Principle

Do not use AI/ML merely because the event has "AI/ML" in its title.

Use the simplest scientifically defensible method.

# 3. TOOL SELECTION

## Google Earth Engine

Use GEE for:

- Satellite data

- Large-area raster processing

- Cloud masking

- Image filtering

- Image composites

- Spectral indices

- Time-series analysis

- Change detection

- Sampling

- Machine learning

- Raster export

### GEE is especially useful when:

- Data is large

- Satellite imagery is required

- Multiple dates are needed

- Processing would be slow locally

# QGIS

Use QGIS for:

- Vector processing

- Raster processing

- Spatial overlay

- Buffering

- Clipping

- Reprojection

- Attribute calculations

- Zonal statistics

- Spatial joins

- Map design

- Layout creation

- Final thematic maps

# Python

Use Python for:

- Data cleaning

- Statistical analysis

- Machine learning

- Charts

- Automation

- Custom algorithms

- Large tabular datasets

- Model comparison

Typical libraries:

pandas

numpy

geopandas

matplotlib

scikit-learn

rasterio

shapely

# 4. DATA SOURCE CHEAT SHEET

## Satellite Data

| Dataset | Main Use |
| --- | --- |
| Sentinel-2 | Optical / vegetation / water / land cover |
| Sentinel-1 | Radar / flood / moisture / change |
| Landsat | Long-term historical analysis |
| MODIS | Large-area / frequent temporal analysis |

## Sentinel-2

Useful bands:

B2  = Blue

B3  = Green

B4  = Red

B5  = Red Edge

B6  = Red Edge

B7  = Red Edge

B8  = NIR

B8A = Narrow NIR

B11 = SWIR 1

B12 = SWIR 2

Approximate resolutions:

10 m → B2, B3, B4, B8

20 m → B5, B6, B7, B8A, B11, B12

60 m → atmospheric bands

## Sentinel-1

Useful for:

- Flood mapping

- Water detection

- Surface moisture

- Structural information

- Monitoring where clouds are a problem

- Before/after disaster comparison

Common concepts:

VV

VH

VV/VH ratio

Backscatter change

## DEM

Possible sources:

- SRTM

- ALOS

- Copernicus DEM

Can derive:

- Elevation

- Slope

- Aspect

- Hillshade

- Terrain ruggedness

- Flow direction

- Drainage-related variables

# 5. OTHER IMPORTANT DATASETS

| Dataset | Possible Use |
| --- | --- |
| OpenStreetMap | Roads, buildings, POIs |
| WorldPop | Population |
| ESA WorldCover | Land cover |
| JRC Global Surface Water | Water occurrence |
| GADM | Administrative boundaries |
| geoBoundaries | Administrative boundaries |
| SRTM | Elevation |
| CHIRPS | Rainfall |
| ERA5 | Weather/climate |
| Field samples | Ground truth |
| Local government data | Validation/context |

# 6. DATA SELECTION RULE

Before downloading anything ask:

### Spatial

- Does it cover the study area?

- Is the resolution sufficient?

### Temporal

- Does it match the problem's time period?

- Do we need before/after imagery?

### Spectral

- Do the available bands support the required analysis?

### Quality

- Cloud-free?

- Correct projection?

- Valid geometry?

- Missing values?

### Relevance

Does this dataset actually help answer the problem?

# 7. COMMON GIS OPERATIONS

## Clip

Keeps only the portion inside a boundary.

Use: Restrict data to study area.

## Buffer

Creates a zone around a feature.

Examples:

Road → 500 m buffer

River → 1 km buffer

Industrial area → 2 km buffer

## Dissolve

Combines features based on a field or completely.

## Intersect

Keeps overlapping portions of layers.

Example:

Flood Area ∩ Buildings

→ Buildings affected by flood.

## Union

Combines polygon layers while preserving boundaries and attributes.

## Difference

Removes one layer's area from another.

## Spatial Join

Transfers attributes based on spatial relationship.

Example:

Points → Administrative districts

## Join Attributes by Location

Attach polygon attributes to points or vice versa.

## Reproject

Changes coordinate reference system.

## Raster Calculator

Perform mathematical operations on raster layers.

Example:

Raster A + Raster B

## Zonal Statistics

Calculate raster statistics inside polygons.

Possible statistics:

- Mean

- Minimum

- Maximum

- Sum

- Standard deviation

## Rasterize

Convert vector → raster.

## Polygonize

Convert raster → vector polygons.

# 8. BUFFER DISTANCE THINKING

Use buffers when the problem involves influence or proximity.

Examples:

Road → pollution influence

River → flood influence

Hospital → accessibility

School → service area

Industry → environmental impact

Settlement → exposure

Coast → coastal vulnerability

Always justify the distance.

Do not choose a buffer randomly.

# 9. REMOTE-SENSING INDICES

## NDVI — Vegetation

[
NDVI = \frac{NIR-Red}{NIR+Red}
]

Sentinel-2:

(NIR - Red) / (NIR + Red)

(B8 - B4) / (B8 + B4)

Use for:

- Vegetation

- Crop health

- Forest condition

- Vegetation change

# NDWI — Water

Common Green-NIR form:

[
NDWI = \frac{Green-NIR}{Green+NIR}
]

Sentinel-2:

(B3 - B8) / (B3 + B8)

Use for:

- Surface water

- Water extent

- Water change

# MNDWI — Modified Water Index

[
MNDWI = \frac{Green-SWIR}{Green+SWIR}
]

Sentinel-2:

(B3 - B11) / (B3 + B11)

Useful for:

- Water detection

- Separating water from built-up surfaces

# NDBI — Built-up

[
NDBI = \frac{SWIR-NIR}{SWIR+NIR}
]

Sentinel-2:

(B11 - B8) / (B11 + B8)

Use for:

- Built-up areas

- Urban expansion

- Settlement mapping

# NDMI — Moisture

[
NDMI = \frac{NIR-SWIR}{NIR+SWIR}
]

Sentinel-2:

(B8 - B11) / (B8 + B11)

Use for:

- Vegetation moisture

- Soil/surface moisture

- Drought-related analysis

# NBR — Burn Severity

[
NBR = \frac{NIR-SWIR2}{NIR+SWIR2}
]

Sentinel-2:

(B8 - B12) / (B8 + B12)

Use for:

- Burned areas

- Fire severity

- Post-fire assessment

# SAVI — Vegetation in Sparse Areas

[
SAVI =
\frac{(NIR-Red)(1+L)}
{NIR+Red+L}
]

Usually:

L = 0.5

Useful when soil strongly affects vegetation measurements.

# EVI — Enhanced Vegetation Index

[
EVI =
2.5
\frac{NIR-Red}
{NIR+6Red-7.5Blue+1}
]

Useful for dense vegetation and reducing some atmospheric/background effects.

# 10. INDEX SELECTION QUICK TABLE

| Target | Index |
| --- | --- |
| Vegetation | NDVI |
| Water | NDWI |
| Water vs built-up | MNDWI |
| Built-up | NDBI |
| Moisture | NDMI |
| Burned area | NBR |
| Dense vegetation | EVI |
| Sparse vegetation | SAVI |

# 11. CHANGE DETECTION

If the problem involves:

- Before vs after

- Expansion

- Reduction

- Damage

- Seasonal change

- Urban growth

- Deforestation

- Flood extent

Use:

Image Before

↓

Preprocess

↓

Image After

↓

Preprocess

↓

Calculate Difference

↓

Threshold / Classify

↓

Validate

↓

Map Change

Possible approaches:

### Difference

After - Before

### Ratio

After / Before

### Index Difference

Example:

NDVI_after - NDVI_before

### Classification Change

Class_before → Class_after

# 12. DISASTER-SPECIFIC QUICK GUIDE

## Flood

Consider:

- Sentinel-1

- NDWI

- MNDWI

- DEM

- Before/after imagery

- Distance to rivers

- Rainfall

Possible workflow:

Sentinel-1

→ Preprocess

→ Water/backscatter analysis

→ Before/after comparison

→ Flood mask

→ Remove permanent water

→ Validate

## Drought

Consider:

- NDVI

- NDMI

- Rainfall

- Temperature

- Long-term vegetation anomaly

## Fire

Consider:

- NBR

- NDVI

- Before/after Sentinel-2

- Burn severity

## Urban Growth

Consider:

- NDBI

- NDVI

- Built-up classification

- Multi-date satellite imagery

- Population

- Roads

## Water Quality / Waterbody Condition

Consider:

- Sentinel-2

- NDWI

- MNDWI

- NIR/SWIR

- Spectral features

- Field samples

- Temporal imagery

Avoid claiming that satellite imagery directly measures a parameter unless scientifically supported and validated.

# 13. FEATURE ENGINEERING

Features are variables given to an analysis/model.

## Spectral Features

Blue

Green

Red

NIR

SWIR1

SWIR2

## Spectral Indices

NDVI

NDWI

MNDWI

NDBI

NDMI

NBR

EVI

SAVI

## Terrain

Elevation

Slope

Aspect

Terrain ruggedness

## Distance

Distance to roads

Distance to rivers

Distance to settlements

Distance to industries

Distance to coast

Distance to hospitals

## Density

Road density

Building density

Population density

POI density

## Temporal

Mean

Minimum

Maximum

Standard deviation

Trend

Before/after difference

Seasonal difference

# 14. FEATURE SELECTION

Do not automatically use every feature.

Ask:

- Is the feature relevant?

- Does it provide unique information?

- Is it available consistently?

- Is it too strongly correlated with another feature?

- Does it improve the model/result?

- Can we explain why we used it?

# 15. AI/ML DECISION TABLE

| Problem | Method to Consider |
| --- | --- |
| Land-cover classification | Random Forest |
| Environmental classification | Random Forest / GTB |
| Non-linear classification | GTB / XGBoost |
| Small labelled dataset | Random Forest / SVM |
| Regression | Random Forest / Gradient Boosting |
| Clustering | K-Means |
| Similarity | Spectral / embedding similarity |
| Anomaly detection | Isolation Forest / statistical methods |
| Time prediction | Regression / time-series methods |

# 16. RANDOM FOREST

Good general-purpose model for:

- Land cover

- Environmental classification

- Multi-class classification

- Non-linear relationships

Advantages:

- Easy to use

- Handles many features

- Works with non-linear relationships

- Usually strong baseline

- Less preprocessing than some models

- Feature importance can be examined

Basic workflow:

Samples

↓

Feature Stack

↓

Train/Test Split

↓

Random Forest

↓

Prediction

↓

Validation

# 17. GRADIENT TREE BOOST

Gradient Tree Boosting is a tree-based ensemble method.

Useful for:

- Classification

- Non-linear relationships

- Complex feature interactions

Important parameters can include:

Number of trees

Learning rate / shrinkage

Sampling rate

Maximum nodes/depth

Loss function

Random seed

Remember:

Google Earth Engine's smileGradientTreeBoost is Gradient Tree Boosting. It is not the same API as Python's XGBoost library.

# 18. XGBOOST

XGBoost is a gradient-boosted decision-tree algorithm.

Useful when:

- You have structured/tabular features

- You have labelled training data

- Non-linear relationships are expected

- You want strong predictive performance

Typical Python library:

xgboost

Do not confuse:

Gradient Tree Boost

with:

XGBoost

They are related boosting approaches but are not identical.

# 19. SUPPORT VECTOR MACHINE

SVM is useful for:

- Classification

- Smaller datasets

- High-dimensional feature spaces

Common kernel:

RBF

Important parameters:

C

Gamma

Kernel

# 20. K-MEANS

Use when labels are unavailable.

Workflow:

Feature data

↓

Choose K

↓

Initialize clusters

↓

Assign points

↓

Update cluster centers

↓

Repeat

↓

Final clusters

Useful for:

- Land-cover exploration

- Regional grouping

- Unsupervised segmentation

Remember:

K-Means clusters data; it does not automatically know what each cluster means.

You must interpret and label the clusters.

# 21. TRAINING DATA

For supervised ML:

Feature data + Known class

Example:

NDVI = 0.72

NDWI = 0.10

NDBI = -0.15

Class = Vegetation

Training data should be:

- Representative

- Correctly labelled

- Spatially distributed

- Sufficient for each class

- Free from obvious errors

# 22. CLASS BALANCE

Avoid:

Class A = 10,000 samples

Class B = 200 samples

if possible.

Try to obtain reasonably balanced training data.

If imbalance is unavoidable:

- Report it

- Consider class weights/resampling

- Use suitable metrics

- Do not rely only on accuracy

# 23. TRAIN/VALIDATION SPLIT

Typical starting point:

80% → Training

20% → Validation

But the exact split depends on the problem.

For spatial data, random pixel splitting can be misleading because nearby pixels are highly similar.

Prefer when appropriate:

Spatially separated training and validation areas

# 24. SPATIAL LEAKAGE

### Problem

Training and validation samples are extremely close or derived from the same spatial patches.

This can make accuracy appear much higher than real-world performance.

### Avoid by:

- Spatial separation

- Independent validation samples

- Separate geographic regions

- Field observations

# 25. CLASSIFICATION VALIDATION

## Confusion Matrix

Predicted

Positive Negative

Actual Positive    TP       FN

Actual Negative    FP       TN

# 26. ACCURACY

[
Accuracy =
\frac{TP+TN}
{TP+TN+FP+FN}
]

Meaning:

Percentage of all predictions that are correct.

# 27. PRECISION

[
Precision =
\frac{TP}
{TP+FP}
]

Meaning:

Of everything predicted as positive, how much was actually positive?

Useful when false positives matter.

# 28. RECALL

[
Recall =
\frac{TP}
{TP+FN}
]

Meaning:

Of all actual positive cases, how many did we detect?

Useful when missing positives is dangerous.

# 29. F1 SCORE

[
F1 =
2\frac{Precision\times Recall}
{Precision+Recall}
]

Useful when both precision and recall matter.

# 30. KAPPA

Kappa measures agreement beyond chance.

It can be reported for classification tasks when appropriate.

Do not rely on Kappa alone.

# 31. REGRESSION VALIDATION

For continuous prediction use:

### MAE

[
MAE =
\frac{1}{n}
\sum |y-\hat y|
]

### RMSE

[
RMSE =
\sqrt{
\frac{1}{n}
\sum(y-\hat y)^2
}
]

### R²

Measures how much variation is explained by the model.

# 32. VALIDATION STRATEGY

Choose according to the problem.

### Classification

Confusion Matrix

Accuracy

Precision

Recall

F1

### Regression

MAE

RMSE

R²

### Spatial mapping

Independent samples

Field data

Reference maps

Visual inspection

Spatial consistency

### Change detection

Before/after validation

Reference imagery

Field observations

Known events

# 33. QGIS QUICK REFERENCE

## Essential Processing Tools

Clip

Buffer

Dissolve

Intersection

Union

Difference

Reproject

Spatial Join

Join Attributes by Location

Fix Geometries

Extract by Location

Zonal Statistics

Raster Calculator

Clip Raster by Mask Layer

Polygonize

Rasterize

Merge

Dissolve

# 34. QGIS MAP DESIGN CHECKLIST

Every final map should normally have:

☐ Clear title
☐ Main thematic layer
☐ Legend
☐ Scale bar
☐ North arrow
☐ Study-area context
☐ Important labels
☐ Coordinate/grid information if useful
☐ Data source
☐ Date
☐ Projection/CRS if relevant
☐ Author/team information if required

Avoid:

- Too many colours

- Huge legends

- Unnecessary decoration

- Excessive labels

- Poor contrast

- Missing units

- Misleading symbology

# 35. THEMATIC MAP COLOUR LOGIC

Use intuitive colours where possible.

| Theme | Typical visual association |
| --- | --- |
| Water | Blue |
| Vegetation | Green |
| Forest | Dark green |
| Agriculture | Green/yellow |
| Built-up | Grey/red |
| Roads | Grey/black |
| Flood | Blue/cyan |
| Fire/burn | Red/orange |
| High risk | Red |
| Medium risk | Orange/yellow |
| Low risk | Green |
| Elevation | Sequential colour ramp |

Use colourblind-friendly palettes when possible.

# 36. RISK MAP

Typical workflow:

Risk Factors

↓

Normalize

↓

Assign Weights

↓

Weighted Overlay

↓

Risk Score

↓

Classify

↓

Validate

↓

Risk Map

Example:

Risk =

0.4 × Hazard

+

0.3 × Exposure

+

0.3 × Vulnerability

Weights must have a logical/scientific justification.

# 37. SUITABILITY ANALYSIS

Common factors:

- Distance to roads

- Distance to water

- Slope

- Land use

- Population

- Existing infrastructure

- Environmental restrictions

Workflow:

Criteria

↓

Normalize

↓

Weight

↓

Weighted Overlay

↓

Suitability Score

↓

Suitable / Unsuitable Classes

# 38. NORMALIZATION

Different variables have different units.

Example:

Distance = metres

Population = persons/km²

Slope = degrees

NDVI = -1 to +1

Convert them to a common scale, often:

0 → unsuitable/low

1 → suitable/high

Possible min-max normalization:

[
X' =
\frac{X-X_{min}}
{X_{max}-X_{min}}
]

For variables where lower values are better, reverse the scale.

# 39. HOTSPOT ANALYSIS

Useful for:

- Crime

- Disease

- Accidents

- Pollution

- Fires

- Traffic

- Incidents

Possible methods:

- Point density

- Kernel Density

- Heatmap

- Getis-Ord Gi*

- Local Moran's I

Do not call a simple heatmap statistically significant hotspot analysis.

# 40. DISTANCE ANALYSIS

Useful for questions like:

How far is each location from the nearest road/hospital/river/school?

Possible QGIS tools:

- Distance to nearest hub

- Proximity raster

- Buffer

- Join by nearest location

# 41. DENSITY ANALYSIS

Examples:

Road density

Building density

Population density

Hospital density

Incident density

Possible methods:

- Kernel density

- Point density

- Line density

- Zonal statistics

# 42. GOOGLE EARTH ENGINE BASIC WORKFLOW

1. Define study area

2. Select dataset

3. Filter dates

4. Filter cloud percentage

5. Apply cloud/shadow mask

6. Create composite

7. Calculate indices

8. Create feature stack

9. Analyze / train model

10. Validate

11. Export result

# 43. SENTINEL-2 CLOUD MASKING

Common SCL classes to consider masking include:

3  = Cloud shadow

8  = Cloud medium probability

9  = Cloud high probability

10 = Cirrus

11 = Snow/ice

Always check the specific dataset documentation and masking strategy.

# 44. GEE FEATURE STACK

Example:

B2

B3

B4

B8

B11

B12

NDVI

NDWI

MNDWI

NDBI

NDMI

NBR

The exact stack should depend on the problem.

# 45. GEE SAMPLING

For supervised classification:

Feature Image

+

Training Polygons/Points

↓

sampleRegions()

↓

Training Table

↓

Train Model

Important fields:

class

feature values

# 46. GEE CLASSIFICATION WORKFLOW

Feature Image

↓

Training Samples

↓

sampleRegions()

↓

Random Split

↓

Train Classifier

↓

Classify Image

↓

Validation

↓

Export

# 47. COMMON GEE CLASSIFIERS

Possible classifiers include:

Random Forest

Gradient Tree Boost

SVM

CART

Naive Bayes

Choose based on the problem and available samples.

# 48. GEE EXPORT CHECKLIST

Before export check:

☐ Correct region
☐ Correct scale
☐ Correct CRS
☐ Correct data type
☐ Correct NoData handling
☐ Correct layer/band
☐ Correct file name
☐ Correct Drive folder

# 49. PYTHON ML WORKFLOW

Typical structure:

Load data

↓

Clean data

↓

Select features

↓

Define target

↓

Train/test split

↓

Train model

↓

Predict

↓

Evaluate

↓

Visualize

↓

Save model/result

# 50. PYTHON DATA SCIENCE QUICK REFERENCE

### pandas

Use for:

CSV

Tables

Filtering

Grouping

Cleaning

### numpy

Use for:

Arrays

Numerical calculations

Matrix operations

### geopandas

Use for:

Shapefiles

GeoJSON

Spatial tables

Vector analysis

### rasterio

Use for:

Raster reading

Raster writing

Raster processing

### matplotlib

Use for:

Charts

Plots

Histograms

Scatter plots

### scikit-learn

Use for:

Machine learning

Preprocessing

Metrics

Train/test split

Models

# 51. DATA CLEANING

Check:

☐ Missing values
☐ Duplicate rows
☐ Incorrect coordinates
☐ Invalid geometries
☐ Wrong CRS
☐ Outliers
☐ Wrong labels
☐ Incorrect units

# 52. COORDINATE REFERENCE SYSTEM

### Geographic CRS

Example:

EPSG:4326

Coordinates are:

Longitude

Latitude

### Projected CRS

Coordinates generally represent:

metres

Useful for:

- Distance

- Area

- Buffer

- Accurate spatial measurements

### Important

Do not calculate distance/area blindly in a geographic CRS when a suitable projected CRS is required.

# 53. COMMON CRS PROBLEM

If layers do not align:

Check:

Layer CRS

Project CRS

Coordinate values

On-the-fly transformation

Do not immediately assume the data is wrong.

# 54. AREA CALCULATION

For meaningful area calculations:

- Use an appropriate projected CRS.

- Calculate geometry area.

- Record units.

Possible units:

m²

hectares

km²

Conversions:

[
1 hectare = 10,000,m^2
]

[
1 km^2 = 1,000,000,m^2
]

# 55. MAP SCALE

Examples:

1:10,000

1:25,000

1:50,000

1:100,000

Smaller study area → generally larger scale.

Larger study area → generally smaller scale.

# 56. SATELLITE RESOLUTION

Remember:

Spatial resolution

→ size of pixel

Temporal resolution

→ how often data is collected

Spectral resolution

→ number/range of spectral bands

Radiometric resolution

→ sensitivity to intensity differences

# 57. PIXEL SIZE WARNING

Do not create false precision.

If source data is 10 m:

10 m data ≠ 1 m accuracy

Resampling to 1 m does not magically create new information.

# 58. THRESHOLDING

Sometimes a problem can be solved with a threshold.

Example:

NDWI > threshold → Water

But:

Thresholds must be justified and validated.

Do not choose a threshold only because it produces a visually attractive map.

# 59. MORPHOLOGICAL / CLEANUP OPERATIONS

Raster classification may contain:

- Noise

- Tiny patches

- Holes

- Isolated pixels

Possible cleanup:

- Sieve

- Majority filter

- Morphological opening/closing

- Connected-component filtering

- Minimum mapping unit

Always ensure cleanup does not remove real small features.

# 60. VECTOR CLEANUP

After raster-to-vector conversion:

Check:

☐ Tiny polygons
☐ Invalid geometries
☐ Slivers
☐ Holes
☐ Duplicate features
☐ Unwanted boundaries

Possible tools:

Fix Geometries

Dissolve

Delete holes

Multipart to singleparts

Simplify

Remove duplicate geometries

# 61. MAP INTERPRETATION

Do not only say:

"The red area is high risk."

Explain:

- Where?

- How much?

- Compared with what?

- Why?

- What factors may explain it?

- How confident are we?

# 62. STATISTICS TO REPORT

Depending on the problem:

### Area

Total area

Class-wise area

Percentage

### Counts

Number of features

Number of affected locations

Number of high-risk zones

### Distance

Mean distance

Minimum distance

Maximum distance

### Population

Exposed population

Population by risk class

### Temporal

Increase

Decrease

Percentage change

# 63. PERCENTAGE CHANGE

[
Percentage\ Change =
\frac{New-Old}{Old}
\times100
]

Example:

Old = 100 km²
New = 120 km²

[
\frac{120-100}{100}\times100
=20%
]

# 64. FINAL MAP STORY

A strong thematic map should answer:

WHERE?

WHAT?

HOW MUCH?

WHY?

WHAT NEXT?

# 65. FINAL PRESENTATION STORY

Use:

Problem

↓

Why it matters

↓

Data

↓

Method

↓

Analysis

↓

AI/ML if used

↓

Validation

↓

Result

↓

Impact

Avoid spending most of the presentation explaining software buttons.

# 66. SINGLE-PAGE MAP / DASHBOARD

Recommended structure:

------------------------------------------------

TITLE

------------------------------------------------

|                                              |

|              MAIN THEMATIC MAP              |

|                                              |

|                                              |

------------------------------------------------

| Key Stats | Findings | Method | Impact       |

------------------------------------------------

Include:

- Main map

- Key statistics

- Important findings

- Short workflow

- Legend

- Data sources

- Impact

# 67. TECHNICAL PRESENTATION

Suggested slides:

1. Title

2. Problem & Objective

3. Study Area

4. Data Sources

5. Methodology

6. Features / Variables

7. AI/ML Model

8. Results

9. Validation

10. Key Findings

11. Impact + Limitations

12. Conclusion

# 68. REPORT STRUCTURE

Keep the final report concise:

1. Problem

2. Objective

3. Study Area

4. Data

5. Methodology

6. Features

7. AI/ML

8. Results

9. Validation

10. Maps

11. Impact

12. Limitations

13. Conclusion

# 69. LIMITATIONS

Always be honest.

Possible limitations:

- Limited field samples

- Cloud contamination

- Satellite resolution

- Temporal mismatch

- Classification uncertainty

- Training data limitations

- Spatial leakage

- Missing datasets

- Model assumptions

- Threshold sensitivity

A limitation does not make the project weak.

Explaining limitations demonstrates scientific understanding.

# 70. COMMON MISTAKES

Avoid:

❌ Using too many datasets
❌ Using unnecessary ML
❌ Randomly choosing thresholds
❌ Claiming unsupported relationships
❌ Training on poor samples
❌ Using the same samples for training and validation
❌ Ignoring spatial leakage
❌ Reporting only accuracy
❌ Ignoring class imbalance
❌ Calculating area incorrectly
❌ Mixing CRS incorrectly
❌ Using inappropriate resolution
❌ Creating visually attractive but scientifically weak maps
❌ Forgetting data sources
❌ Forgetting validation
❌ Making unsupported conclusions

# 71. SCIENTIFIC CLAIM CHECK

Before writing a conclusion ask:

### Evidence

What data supports this claim?

### Method

What analysis produced this result?

### Validation

How did we verify it?

### Uncertainty

What could be wrong?

If you cannot answer these, weaken the claim.

# 72. DATA → FEATURE → MODEL → MAP

Remember this relationship:

RAW DATA

↓

PREPROCESSING

↓

FEATURES

↓

MODEL / ANALYSIS

↓

PREDICTION

↓

VALIDATION

↓

THEMATIC MAP

Example:

Sentinel-2

↓

Cloud masking

↓

B2 B3 B4 B8 B11 B12

NDVI NDWI MNDWI NDBI

↓

Random Forest

↓

Land-cover classes

↓

Validation

↓

Land-cover map

# 73. IF YOU HAVE FIELD SAMPLES

Field samples can be used for:

- Training

- Validation

- Ground truth

- Spectral reference

- Independent verification

- Accuracy assessment

But decide their role carefully.

Do not automatically use every sample for training.

# 74. SMALL SAMPLE WARNING

If you have very few field samples:

Avoid unnecessarily complex supervised ML.

Consider:

- Independent validation

- Spectral similarity

- Reference-based methods

- Simpler models

- Additional reference datasets

- Careful interpretation

The model should match the amount and quality of available evidence.

# 75. SPECTRAL SIMILARITY

If labelled samples are limited:

Reference sample spectrum

↓

Compare with image pixels

↓

Similarity score

↓

Threshold / ranking

↓

Potential matching areas

Possible approaches:

- Euclidean distance

- Spectral Angle Mapper

- Correlation

- Embedding similarity

# 76. SATELLITE EMBEDDINGS

Satellite embeddings can represent image information in a feature space.

Possible workflow:

Satellite Embedding

↓

Reference samples

↓

Similarity calculation

↓

Similarity surface

↓

Map potential matching areas

Useful when:

- You have limited labels

- Direct classification is difficult

- You want similarity to known examples

# 77. MODEL COMPARISON

If time permits:

Model A

↓

Validation

Model B

↓

Validation

Compare

↓

Choose best scientifically justified model

Do not choose a model only because it has the highest training accuracy.

Prefer strong validation performance and interpretability.

# 78. HYPERPARAMETER RECORD

Whenever using ML, record:

Model:

Number of trees:

Learning rate:

Depth / max nodes:

Sampling rate:

Features:

Training samples:

Validation samples:

Train/test split:

Random seed:

Threshold:

This makes the work reproducible.

# 79. REPRODUCIBILITY CHECKLIST

Before final submission:

☐ Code saved
☐ Data source recorded
☐ Date recorded
☐ Parameters recorded
☐ CRS recorded
☐ Processing steps recorded
☐ Training samples saved
☐ Validation samples saved
☐ Final map exported
☐ Final raster/vector saved
☐ Report saved
☐ Presentation saved

# 80. FILE NAMING

Use simple names.

Good:

sentinel2_composite.tif

feature_stack.tif

training_samples.shp

validation_samples.shp

rf_classification.tif

gtb_classification.tif

final_thematic_map.pdf

final_thematic_map.png

results_report.docx

final_presentation.pptx

Avoid:

finalfinal2_new_latest_reallyfinal.tif

# 81. MAPATHON TEAM WORKFLOW

If working as a team:

### Person 1 — Data

- Satellite

- Boundaries

- Supporting datasets

### Person 2 — Analysis

- Feature engineering

- GIS analysis

- ML

### Person 3 — QGIS

- Spatial processing

- Map layout

- Symbology

### Person 4 — Documentation

- Results

- Statistics

- Presentation

- Report

Roles can overlap.

# 82. TEAM FILE WORKFLOW

Use:

RAW

↓

PROCESSED

↓

ANALYSIS

↓

DRAFT

↓

FINAL

Never overwrite important raw data.

# 83. 24-HOUR MAPATHON PRIORITY

When time is limited:

### Priority 1

Get a correct result.

### Priority 2

Validate it.

### Priority 3

Create the main map.

### Priority 4

Calculate useful statistics.

### Priority 5

Improve visualization.

### Priority 6

Prepare presentation/report.

Do not spend three hours designing the map before knowing whether the analysis works.

# 84. EMERGENCY SIMPLIFICATION

If the original approach is failing:

Complex ML

↓

Simpler ML

↓

Statistical method

↓

Rule-based GIS

A simple correct solution is better than a complex broken solution.

# 85. IF DATA IS MISSING

Do not immediately panic.

Try:

Official dataset

↓

Open-source dataset

↓

GEE dataset

↓

OSM

↓

Existing reference dataset

↓

Derived satellite feature

If a required dataset genuinely cannot be obtained:

- Document it

- Find a defensible substitute

- State the limitation

# 86. IF THE MODEL PERFORMS POORLY

Check:

Training samples

↓

Labels

↓

Features

↓

Class imbalance

↓

Cloud/noise

↓

Spatial leakage

↓

Model choice

↓

Hyperparameters

↓

Validation design

Do not immediately increase model complexity.

# 87. IF ACCURACY IS SUSPICIOUSLY HIGH

For example:

98–100%

Ask:

- Is there spatial leakage?

- Are training and validation samples overlapping?

- Are the samples too close?

- Is the validation data independent?

- Is the target leaking into the features?

- Is the class distribution misleading?

Very high accuracy is not automatically good evidence.

# 88. FINAL QUALITY CHECK

## Data

☐ Correct data
☐ Correct date
☐ Correct resolution
☐ Correct CRS
☐ Good quality

## Analysis

☐ Relevant features
☐ Appropriate method
☐ Reasonable parameters
☐ No obvious leakage
☐ Reproducible

## Validation

☐ Independent validation
☐ Appropriate metrics
☐ Confusion matrix if applicable
☐ Limitations documented

## Map

☐ Clear title
☐ Correct legend
☐ Correct colours
☐ Scale bar
☐ North arrow
☐ Labels
☐ Sources

## Presentation

☐ Problem clear
☐ Method clear
☐ Results clear
☐ Validation shown
☐ Impact explained

# 89. ONE-MINUTE MAPATHON DECISION TREE

When the problem is announced:

WHAT IS THE TARGET?

↓

Detection / Classification / Prediction /

Change / Risk / Suitability / Hotspot

↓

WHAT DATA?

↓

Satellite / DEM / OSM / Population /

Weather / Field Data

↓

WHAT TOOL?

↓

GEE / QGIS / Python

↓

WHAT FEATURES?

↓

Bands / Indices / Terrain /

Distance / Density / Temporal

↓

DO WE NEED ML?

↓

NO → GIS / statistical method

YES

↓

Classification → RF / GTB / SVM

Regression → RF / Gradient Boosting

Clustering → K-Means

Similarity → Spectral / Embedding

↓

VALIDATE

↓

ACCURACY / F1 / RMSE / FIELD DATA

↓

FINAL THEMATIC MAP

↓

STATISTICS

↓

PRESENTATION

↓

REPORT

# 90. FINAL MAPATHON GOLDEN RULES

### Rule 1

Understand the problem before touching the data.

### Rule 2

Use the minimum data necessary to answer the question.

### Rule 3

Choose the simplest scientifically defensible method.

### Rule 4

AI/ML is a tool, not the objective.

### Rule 5

Good training data is more important than a complicated model.

### Rule 6

Never trust model accuracy without checking validation design.

### Rule 7

Avoid spatial leakage.

### Rule 8

Every important claim should have evidence.

### Rule 9

A thematic map should communicate a decision, not merely display colours.

### Rule 10

Save intermediate results and code.

### Rule 11

Record parameters so the result can be reproduced.

### Rule 12

When time is short, prioritize correctness → validation → map → presentation.

# 91. FINAL EMERGENCY CHECKLIST

Before submission:

☐ Problem understood
☐ Objective defined
☐ Study area defined
☐ Data collected
☐ Data preprocessed
☐ Features created
☐ Method selected
☐ ML trained if required
☐ Validation completed
☐ Accuracy calculated
☐ Results interpreted
☐ Statistics calculated
☐ Main thematic map completed
☐ Supporting map completed
☐ Sources documented
☐ Limitations documented
☐ Presentation completed
☐ Report completed
☐ Code backed up
☐ Final files backed up

# END

## Core Workflow to Remember

PROBLEM → DATA → PREPROCESS → FEATURES → ANALYSIS/ML → VALIDATE → MAP → INTERPRET → PRESENT

Correctness first. Validation second. Visualization third. Complexity last.